clc; clear; close all; % Experiment 1: Butterworth IIR Filters Fs = 8000; % Sampling frequency (Hz) Ap = 1; % Passband attenuation (dB) As = 40; % Stopband attenuation (dB) % Low-Pass Filter Specifications fp_lp = 1000; fs_lp = 1500; wp_lp = fp_lp/(Fs/2); ws_lp = fs_lp/(Fs/2); [N_lp, Wn_lp] = buttord(wp_lp, ws_lp, Ap, As); [b_lp, a_lp] = butter(N_lp, Wn_lp, 'low'); % High-Pass Filter Specifications fp_hp = 2500; fs_hp = 1800; wp_hp = fp_hp/(Fs/2); ws_hp = fs_hp/(Fs/2); [N_hp, Wn_hp] = buttord(wp_hp, ws_hp, Ap, As); [b_hp, a_hp] = butter(N_hp, Wn_hp, 'high'); % Display Results fprintf('BUTTERWORTH IIR FILTER DESIGN\n\n');

BUTTERWORTH IIR FILTER DESIGN

fprintf('LOW-PASS FILTER\n');

LOW-PASS FILTER

fprintf('Filter Order = %d\n', N_lp);

Filter Order = 12

fprintf('Cutoff Frequency = %.2f Hz\n', Wn_lp*Fs/2);

Cutoff Frequency = 1087.84 Hz

fprintf('Numerator Coefficients:\n');

Numerator Coefficients:

# disp(b_lp);

0.0000    0.0000    0.0002    0.0006    0.0013    0.0021    0.0024    0.0021    0.0013    0.0006

0.

fprintf('Denominator Coefficients:\n');

Denominator Coefficients:

# disp(a_lp);

1.0000   -5.4622   14.6771  -25.1616   30.3497  -26.9537   17.9862   -9.0534    3.4018   -0.9283

0.

fprintf('\nHIGH-PASS FILTER\n');

HIGH-PASS FILTER

fprintf('Filter Order = %d\n', N_hp);

Filter Order = 10

fprintf('Cutoff Frequency = %.2f Hz\n', Wn_hp*Fs/2);

Cutoff Frequency = 2379.76 Hz

fprintf('Numerator Coefficients:\n');

Numerator Coefficients:

# disp(b_hp);

0.0006   -0.0055    0.0248   -0.0662    0.1159   -0.1390    0.1159   -0.0662    0.0248   -0.0055    0.

fprintf('Denominator Coefficients:\n');

Denominator Coefficients:

# disp(a_hp);

1.0000    1.8915    2.8535    2.6014    1.8704    0.9534    0.3726    0.1030    0.0199    0.0023

0.

% Frequency Response [H_lp, f] = freqz(b_lp, a_lp, 2048, Fs); [H_hp, ~] = freqz(b_hp, a_hp, 2048, Fs); % Plot Results figure('Name','Butterworth IIR Filters'); subplot(2,2,1); plot(f, 20*log10(max(abs(H_lp),1e-8)), 'b', 'LineWidth',1.5); grid on; title('Low-Pass Filter Magnitude Response'); xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

xlim([0 Fs/2]); subplot(2,2,2); plot(f, unwrap(angle(H_lp))*180/pi, 'r', 'LineWidth',1.5); grid on; title('Low-Pass Filter Phase Response'); xlabel('Frequency (Hz)'); ylabel('Phase (degrees)'); xlim([0 Fs/2]); subplot(2,2,3); plot(f, 20*log10(max(abs(H_hp),1e-8)), 'b', 'LineWidth',1.5); grid on; title('High-Pass Filter Magnitude Response'); xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]); xlim([0 Fs/2]); subplot(2,2,4); plot(f, unwrap(angle(H_hp))*180/pi, 'r', 'LineWidth',1.5); grid on; title('High-Pass Filter Phase Response'); xlabel('Frequency (Hz)'); ylabel('Phase (degrees)'); xlim([0 Fs/2]);